# Detailed Design Document

Juniper Mist AI Network

| Client: | [CLIENT] |
| --- | --- |
| Date: | November 28, 2025 |
| Consulting Company: | [Consulting Company Name] |
| Submitted By: | [Name ¦ Email ¦ Phone] |
| Opportunity No: | OPP-XXXXX |
| Version: | 1.0 |

# Table of Contents

1 Introduction
    1.1 Purpose
        1.1.1 Scope
2 Table Content
3 Image Section
4 Additional Content
5 Sign-Off

# List of Figures

Figure 1: Sample Architecture Diagram

# List of Tables

Table 1: Sample Deliverables Table

# 1 Executive Summary

This document provides the comprehensive technical design for the Juniper Mist AI Network implementation. The solution delivers AI-powered wireless infrastructure with 50 WiFi 6E access points, cloud-managed switching, and intelligent network analytics to achieve 99.9% uptime and 90% reduction in troubleshooting time.

## 1.1 Purpose

Define the technical architecture and design specifications for deployment of Juniper Mist AI wireless network, including 50 AP45 WiFi 6E access points, 4 EX4400 PoE switches, Marvis virtual assistant, location services with vBLE, and unified cloud management through Mist dashboard.

## 1.2 Scope

In-scope:

• 50 Juniper AP45 WiFi 6E access points (9.6 Gbps tri-band throughput)

• 4 EX4400 PoE switches with Wired Assurance

• Mist Cloud AI platform with predictive analytics

• Marvis Virtual Network Assistant for conversational troubleshooting

• Location services with virtual Bluetooth LE (vBLE)

• 802.1X RADIUS authentication with WPA3 encryption

• SSID configuration for 3-5 wireless networks

• SIEM integration for security event monitoring

Out-of-scope:

• Hardware procurement (client responsibility)

• Structured cabling installation for AP mounting

• Legacy wireless controller decommissioning

• Custom mobile application development beyond standard wayfinding

## 1.3 Assumptions & Constraints

The following assumptions underpin the design and must be validated during implementation.

• Client provides building floor plans in CAD or PDF format for RF planning

• Structured cabling and PoE power available at AP mounting locations

• Network infrastructure (VLANs, DHCP, DNS) configured for wireless requirements

• RADIUS or Active Directory accessible for 802.1X authentication

• Internet connectivity available for Mist Cloud management and AI services

## 1.4 References

This document should be read in conjunction with the following related materials.

• Statement of Work (SOW) - Project scope and investment

• Solution Briefing - Architecture overview and business case

• Juniper Mist AI Technical Documentation

• Client wireless policies and compliance requirements

# 2 Business Context

This section establishes the business drivers, success criteria, and compliance requirements that shape the technical design decisions for the Mist AI Network implementation.

## 2.1 Business Drivers

The solution addresses the following key business objectives identified during discovery.

• Eliminate Manual Troubleshooting: Replace hours of packet captures with AI-driven root cause analysis, achieving 90% reduction in troubleshooting time

• Achieve 99.9% Uptime: Deploy cloud-managed WiFi 6E with proactive anomaly detection before user impact

• Enable Location Services: Implement vBLE for indoor wayfinding and asset tracking without physical beacon hardware

• Reduce Helpdesk Tickets: Achieve 60% reduction in wireless helpdesk tickets through Marvis AI self-service

## 2.2 Workload Criticality & SLA Expectations

The following service level targets define the operational requirements for the production environment.

| Metric | Target | Measurement | Priority |
| --- | --- | --- | --- |
| Wireless Uptime | 99.9% | Mist Cloud SLE | Critical |
| Successful Connections | 99% SLE | Mist analytics | Critical |
| RF Coverage | -67 dBm minimum | Heat mapping | Critical |
| Location Accuracy | 3-5 meters | vBLE positioning | High |
| Roaming Performance | < 150ms handoff | Client testing | High |

Table 1: Service Level Targets

## 2.3 Compliance & Regulatory Factors

The solution must adhere to the following regulatory and compliance requirements.

• SOC 2 Type II compliance for security controls and audit logging

• WPA3-Enterprise encryption for wireless data protection

• 802.1X authentication for user access control

• Comprehensive logging for security event correlation

## 2.4 Success Criteria

Project success will be measured against the following criteria at go-live.

• 50 AP45 access points operational with complete building coverage

• 99% successful connection SLE achieved as measured by Mist Cloud

• Marvis AI operational with proactive anomaly detection

• Location services enabled with indoor positioning within 3-5 meters

• 802.1X authentication functional with RADIUS integration

# 3 Current-State Assessment

This section documents the existing wireless infrastructure that the Mist AI platform will replace.

## 3.1 Application Landscape

The current wireless infrastructure consists of legacy controller-based WiFi requiring modernization.

| Component | Purpose | Technology | Status |
| --- | --- | --- | --- |
| Wireless Controller | Central management | Legacy on-premises | To be replaced |
| Access Points | WiFi coverage | Mixed vendor 802.11ac | To be replaced |
| Guest Portal | Visitor access | Standalone appliance | To be consolidated |
| RF Management | Channel optimization | Manual tuning | To be automated |

Table 2: Solution Components

## 3.2 Infrastructure Inventory

The current infrastructure consists of the following components being replaced.

| Component | Quantity | Specifications | Notes |
| --- | --- | --- | --- |
| Legacy APs | ~40 | 802.11ac Wave 2 | End of support approaching |
| Controller | 1 | On-premises appliance | Single point of failure |
| Guest Portal | 1 | Standalone server | Manual management |
| RF Tools | Various | Manual site survey | No AI optimization |

Table 3: Infrastructure Inventory

## 3.3 Dependencies & Integration Points

The current environment has the following external dependencies.

• Active Directory for user authentication

• RADIUS server for 802.1X integration

• DHCP servers for IP address allocation

• DNS servers for name resolution

• Splunk SIEM for security logging

## 3.4 Network Topology

Current network uses hub-and-spoke wireless topology with:

• Controller-based architecture with centralized data plane

• Limited visibility into client experience

• Manual RF channel and power configuration

• No location services capability

## 3.5 Performance Baseline

Current system performance metrics establish the baseline for improvement.

• Wireless coverage: 85% of building area (gaps in conference rooms)

• Connection success rate: ~92% (manual measurement)

• RF optimization: Manual (quarterly at best)

• Troubleshooting time: 2-4 hours per incident

• Helpdesk tickets: 50+ wireless tickets per month

# 4 Solution Architecture

The target architecture deploys Juniper Mist AI wireless platform with cloud-managed intelligence, automated RF optimization, and comprehensive analytics.

## 4.1 Architecture Principles

The following principles guide all architectural decisions.

• AI-First Design: Leverage machine learning for anomaly detection and optimization

• Cloud-Native Management: Eliminate on-premises controllers for simplified operations

• Zero-Touch Provisioning: Automated AP configuration via Mist Cloud

• Client Experience Focus: SLE-based monitoring for proactive issue detection

• Location Awareness: Built-in vBLE for indoor positioning without hardware beacons

## 4.2 Architecture Patterns

The solution implements the following architectural patterns.

• Cloud-Managed Pattern: All management and analytics via Mist Cloud

• Distributed Data Plane: Traffic forwarding at AP level (no controller hairpin)

• AI Analytics Pattern: Machine learning for RF optimization and anomaly detection

• Microservices Integration: RESTful APIs for external system integration

## 4.3 Component Design

The solution comprises the following logical components.

| Component | Purpose | Technology | Dependencies | Scaling |
| --- | --- | --- | --- | --- |
| Access Points | WiFi 6E coverage, 9.6 Gbps | AP45 (50 units) | PoE switches | Per-area |
| PoE Switches | Power and connectivity | EX4400 (4 units) | Uplinks | Per-closet |
| Mist Cloud | Management and AI | SaaS platform | Internet | Elastic |
| Marvis VNA | Conversational troubleshooting | AI assistant | Mist Cloud | Included |
| Location Services | Indoor positioning | vBLE (built-in) | AP45 vBLE | Per-AP |

Table 4: Solution Components

## 4.4 Technology Stack

The technology stack has been selected based on performance, AI capabilities, and alignment with organizational standards.

| Layer | Technology | Rationale |
| --- | --- | --- |
| Access Points | Juniper AP45 WiFi 6E | 9.6 Gbps tri-band, vBLE integrated |
| Switching | EX4400-48P-POE | Full PoE++, Wired Assurance enabled |
| Cloud Platform | Mist Cloud AI | ML-driven analytics, zero controllers |
| AI Assistant | Marvis VNA | Natural language troubleshooting |
| Authentication | 802.1X with RADIUS | Enterprise security, WPA3 encryption |

Table 5: Technology Stack

# 5 Security & Compliance

This section details the security controls, compliance mappings, and governance mechanisms.

## 5.1 Identity & Access Management

Access control follows enterprise standards with centralized authentication.

• User Authentication: 802.1X with RADIUS server integration

• Admin Authentication: SSO with role-based access control

• Encryption: WPA3-Enterprise with AES-256-GCM

• Guest Access: Captive portal with terms acceptance

### 5.1.1 Role Definitions

The following roles define access levels within Mist Cloud.

| Role | Permissions | Scope |
| --- | --- | --- |
| Super User | Full system access, all configuration | Organization-wide |
| Network Admin | Site configuration, AP management | Assigned sites |
| Helpdesk | Read-only, basic troubleshooting | All sites |
| Observer | Dashboard viewing only | Assigned sites |

Table 6: Role Permissions

## 5.2 Network Security

Network security implements multiple layers of wireless protection.

• SSID Segmentation: Separate networks for corporate, guest, IoT

• VLAN Isolation: Traffic separation between user groups

• Rogue AP Detection: Automatic identification of unauthorized APs

• WPA3 Encryption: Latest WiFi security standard

## 5.3 Data Protection

Data protection controls ensure confidentiality for wireless traffic.

• Encryption in Transit: WPA3-Enterprise for client traffic

• Management Security: TLS 1.3 for cloud communication

• Data Residency: Mist Cloud data stored in US data centers

• Privacy Controls: MAC randomization support for client privacy

## 5.4 Compliance Mappings

The following table maps compliance requirements to implementation controls.

| Framework | Requirement | Implementation |
| --- | --- | --- |
| SOC 2 | Access control | 802.1X with role-based authorization |
| SOC 2 | Audit logging | Comprehensive event logging to SIEM |
| SOC 2 | Encryption | WPA3-Enterprise with AES-256 |
| PCI DSS | Network segmentation | VLAN isolation for payment systems |

Table 7: Compliance Requirements

## 5.5 Audit Logging & SIEM Integration

Comprehensive audit logging supports security monitoring and compliance.

• All authentication events logged with user identity

• Configuration changes captured with administrator audit trail

• Security events forwarded to Splunk SIEM via syslog

• Mist Cloud provides 30-day event retention

• SIEM retention per organizational policy

# 6 Data Architecture

This section defines the data flow, analytics architecture, and RF data management.

## 6.1 Data Model

### 6.1.1 Conceptual Model

The solution manages the following core data elements:

• Client Analytics: Connection statistics, roaming events, throughput

• RF Data: Signal strength, interference, channel utilization

• Location Data: vBLE positioning, asset tracking, occupancy

• Configuration Data: SSID profiles, policies, AP settings

### 6.1.2 Logical Model

The logical data model defines the primary entities.

| Entity | Key Attributes | Relationships | Volume |
| --- | --- | --- | --- |
| Client Device | MAC, hostname, OS | Connects to SSID | 2,000 devices |
| Access Point | Serial, location, RF | Serves clients | 50 APs |
| SSID Profile | Name, security, VLAN | Applied to APs | 3-5 SSIDs |
| Location Event | Timestamp, coordinates | References device | 100K/day |

Table 8: Data Model Entities

## 6.2 Data Flow Design

1. Client Connection: Device associates with nearest AP based on RF signal

2. Authentication: 802.1X validates credentials against RADIUS

3. Traffic Flow: Data forwarded directly from AP (no controller tunnel)

4. Analytics Collection: Mist Cloud ingests telemetry every 60 seconds

5. AI Processing: Machine learning analyzes patterns for anomalies

## 6.3 Data Governance

Data governance policies ensure proper handling of wireless analytics.

• Classification: Client MAC addresses treated as PII

• Retention: Mist Cloud retains 30 days; archive per policy

• Privacy: Support for MAC randomization and data anonymization

• Access: Role-based access to analytics dashboards

# 7 Integration Design

This section documents the integration patterns and external system connections.

## 7.1 External System Integrations

The solution integrates with the following external systems.

| System | Type | Protocol | Format | Error Handling | SLA |
| --- | --- | --- | --- | --- | --- |
| RADIUS | Real-time | RADIUS | Native | Failover server | 99.9% |
| Active Directory | Real-time | LDAP | Native | Cache credentials | 99.9% |
| Splunk SIEM | Real-time | Syslog/TLS | CEF | Buffer and retry | 99.9% |
| ServiceNow | Webhook | HTTPS | JSON | Retry with backoff | 99.5% |

Table 9: System Integrations

## 7.2 API Design

Mist Cloud provides RESTful APIs for automation and integration.

• Style: RESTful with JSON payloads

• Authentication: API token with organization scope

• Rate Limiting: 5000 requests per hour

• Documentation: OpenAPI specification available

### 7.2.1 API Endpoints

The following REST API endpoints provide programmatic access.

| Method | Endpoint | Auth | Description |
| --- | --- | --- | --- |
| GET | /api/v1/sites | API Token | List managed sites |
| GET | /api/v1/sites/{id}/stats/clients | API Token | Client statistics |
| POST | /api/v1/sites/{id}/wlans | API Token | Create SSID |
| GET | /api/v1/orgs/{id}/inventory | API Token | Device inventory |

Table 10: API Endpoints

## 7.3 Authentication & SSO Flows

Single sign-on and authentication flows for administration.

• SSO integration with Okta, Azure AD, or Google Workspace

• SAML 2.0 for administrator authentication

• API tokens for automation and scripting

• MFA enforcement for privileged access

# 8 Infrastructure & Operations

This section covers the infrastructure design and operational procedures.

## 8.1 Network Design

The network architecture provides WiFi 6E coverage across the entire building.

• Access Layer: 50 AP45 access points across all floors

• Distribution: 4 EX4400-48P switches in IDF closets

• Management: Mist Cloud via internet connectivity

• Redundancy: Dual uplinks from each IDF to core

## 8.2 Compute Sizing

Hardware sizing based on coverage requirements and user density.

| Component | Model | Capacity | Coverage | Count |
| --- | --- | --- | --- | --- |
| Access Point | AP45 | 9.6 Gbps, 500 clients | 5,000 sq ft | 50 |
| PoE Switch | EX4400-48P | 48 ports, 1440W PoE | 12 APs each | 4 |
| Mist Cloud | SaaS | Unlimited | N/A | 1 org |

Table 11: Component

## 8.3 High Availability Design

The solution provides resilience through distributed architecture.

• No single point of failure (no central controller)

• AP operates independently if cloud connectivity lost

• Dual power supplies on EX4400 switches

• Geographic redundancy in Mist Cloud

## 8.4 Monitoring & Alerting

Comprehensive monitoring provides visibility across wireless infrastructure.

• SLE Monitoring: Successful connects, coverage, capacity, roaming

• RF Analytics: Channel utilization, interference, RSSI distribution

• Client Experience: Throughput, latency, application performance

• Alerting: Webhook, email, and Slack integration

### 8.4.1 Alert Definitions

The following alerts are configured for proactive monitoring.

| Alert | Condition | Severity | Response |
| --- | --- | --- | --- |
| AP Offline | No heartbeat 5 min | Critical | Check power/network |
| SLE Below Target | < 95% success | Warning | Investigate RF |
| High Interference | Channel busy > 50% | Warning | Adjust RF policy |
| Rogue AP Detected | Unknown BSSID | High | Security review |

Table 12: Monitoring Alerts

## 8.5 Cost Model

The infrastructure costs align with the project investment summary.

| Category | Year 1 Investment | Annual Recurring | Notes |
| --- | --- | --- | --- |
| Hardware | $116,600 | $0 | 50 AP45, 4 EX4400 |
| Software Licenses | $31,400 | $31,400 | Mist subscriptions |
| Support | $9,720 | $9,720 | 24x7 JTAC |
| Professional Services | $81,000 | $0 | Implementation |

Table 13: Category And Year 1 Investment

# 9 Implementation Approach

This section outlines the deployment strategy and sequencing.

## 9.1 Migration/Deployment Strategy

The deployment strategy phases AP installation with legacy cutover.

• Approach: Parallel deployment with phased legacy AP removal

• Pattern: Floor-by-floor rollout with RF validation

• Validation: SLE monitoring before each floor cutover

• Rollback: Legacy APs remain operational until validation complete

## 9.2 Sequencing & Wave Planning

The implementation follows an 8-week phased approach.

| Phase | Activities | Duration | Exit Criteria |
| --- | --- | --- | --- |
| Phase 1 | Discovery & Planning | Weeks 1-2 | RF design approved |
| Phase 2 | Infrastructure Deployment | Weeks 3-5 | All APs operational |
| Phase 3 | AI Services Enablement | Week 6 | Marvis AI active |
| Phase 4 | Testing & Handover | Weeks 7-8 | 99% SLE validated |

Table 14: Implementation Phases

## 9.3 Tooling & Automation

The following tools provide the automation foundation.

| Category | Tool | Purpose |
| --- | --- | --- |
| Cloud Management | Mist Dashboard | Configuration and monitoring |
| RF Planning | Mist AI Planner | Predictive coverage modeling |
| Troubleshooting | Marvis VNA | Conversational diagnostics |
| API Automation | Mist REST API | Scripted configuration |

Table 15: Tools and Technologies

## 9.4 Cutover Approach

The cutover strategy minimizes user impact through phased migration.

• Type: Phased floor-by-floor cutover

• Duration: 2-3 days per floor during Phase 2

• Validation: SLE confirmation before legacy AP removal

• Rollback: Re-enable legacy AP if issues detected

# 10 Appendices

## 10.1 Naming Conventions

All Mist resources follow standardized naming conventions.

| Resource Type | Pattern | Example |
| --- | --- | --- |
| Site | `{building}-{location}` | `hq-newyork` |
| Access Point | `{floor}-ap-{sequence}` | `f1-ap-001` |
| SSID | `{company}-{purpose}` | `acme-corp`, `acme-guest` |
| RF Template | `{density}-{band}` | `high-density-6ghz` |

Table 16: Naming Conventions

## 10.2 Risk Register

The following risks have been identified with mitigation strategies.

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| RF coverage gaps | Medium | High | Predictive site survey, post-deploy validation |
| Hardware delivery delays | Low | Medium | Order immediately upon SOW execution |
| RADIUS integration issues | Low | Medium | Early authentication testing |
| User adoption challenges | Low | Low | Comprehensive training program |

Table 17: Risk Register

## 10.3 Glossary

The following terms are used throughout this document.

| Term | Definition |
| --- | --- |
| AP45 | Juniper WiFi 6E access point with 9.6 Gbps throughput |
| EX4400 | Juniper PoE switch with Wired Assurance support |
| Marvis | AI virtual network assistant for troubleshooting |
| SLE | Service Level Expectation - Mist success metrics |
| vBLE | Virtual Bluetooth LE for location without beacons |
| WiFi 6E | 802.11ax with 6 GHz band support |

Table 18: Glossary

eoframework.org